���������������������������������������������������������������Ŀ

�                                                               �

�    PCOM.EXE  -  Communications & File Transfer Utility        �

�                 For ATARI Portfolio (R).                      �

�                                                               �

�                 By Ender KOROGLU, (C) 1992                    �

�                                                               �

�����������������������������������������������������������������

Using, copying, distributing of this program is FREE ($0.00).

However, selling or modifying is PROHIBITED.

1> TERMINAL-MODE: �����������������������������������������������

This is a Communications & File Transfer utility to be used

as a handy terminal for your BBS or Electronic-Mail needs

while you are on a business/recreational trips. It has user

friendly menus and file capture feature. It can D/U-Loads

any binary/Ascii file Fm/To-Host or another computer using

XMODEM-CheckSum protocol with 1200/300 baud.

Program also Up-Loads Ascii-Text file with 1200/300 baud and

Dn-Loads with 300 baud using ASCII protocol (NO PROTOCOL..).

Program has File-Capture feature to save the contents of a

terminal communications session or an Ascii-Text file while

downloading with 1200 baud.

F5 ......... Appending   Mode for file capturing. (Toggle)

Shift+F5 ... OverWritten Mode  "    "      "         "

Program has 100 lines (100x38 characters) scrollback feature

to monitor a terminal communications session.

F8 ......... Displays Buffer-Content.

Shift+F8 ... Clears     "      "

Communications parameters; 1200/300, N/O/E, 8/7, 1/2.

2> DIRECT-FILE-TRANSFER-MODE: �����������������������������������

Program can send/receive any text/binary file from/to another

computer (Portfolio or PC..) using 3-wire serial cable via

serial interface. For this option; you must use 'PCOM.EXE' on

both side. Because it uses special batch file transferring

protocol named as 'MEK-Protocol'. This method makes possible

file transferring with the exact file size. Any wildcard

characters may be used for batch transferring.

Communications parameters; 2400/1200, N, 8, 1.

3> SETUP-MODE: ��������������������������������������������������

Program parameters may be changed in accordance with your

hardware requirements or with remote computer/BBS software

and be saved as PCOM.INI file. Such as;

- Baud-Rate ..... 2400 / 1200 / 300

- Parity ........ (N)one / (O)dd / (E)ven

- Data-Bits ..... 8 / 7

- Stop-Bits ..... 1 / 2

- Echo .......... (L)ocal / (R)emote / (B)oth / (N)one

and the other settings are as follows;

- Dial-Prefix-String, Hang-Up-String, Auto-Answer-String.

- Dial-Phone-#, Macro-Keys-1..4 (F1..F4), Capture-File-Name.

4> MISCELLANOUS: ������������������������������������������������

- This program has been designed to be used on ATARI's palmtop

computers named PORTFOLIO (R).

- This program has been coded in C language and compiled

using BORLAND's Turbo C compiler.

- This program has been compressed using PKWARE's PKLITE.EXE

and archived by PKZIP.EXE.

THANKS FOR ALL THESE VENDORS FOR CREATING SUCH NICE PRODUCTS..

5> REMARKS: �����������������������������������������������������

Please note that; the parent program 'MEK-Com' (MCOM.EXE) may

be available which is designed to be used on PC-XT/AT & PS/2

computers actually. (Fully compatible with this program..)

If you use this program and wish to transmit your comments

to me; please leave an E-Mail to my address on 'CompuServe'

or 'BBS's' or send a fax-message or write to my office.

Ender KOROGLU, (Elec. Eng. M.S.)

Istanbul/TURKEY.

a) CompuServe E-Mail address ......... 100021,2514

b) SoftComm BBS # (90-1) 380-9938 .... Ender Koroglu

Skylight BBS # (90-1) 337-9393 .... Ender Koroglu

c) Fax-# ............................. (90-1) 264-5739

Modem-# (on request) .............. (90-1) 268-5031

d) Address ................... Mr. ENDER KOROGLU

2.ULUS, YOL S. KIR A-BL, D:5

LEVENT 80600 ISTANBUL, TURKEY

- * * * -